Online Resource 1

Cross-study generalization of machine-learning models for UCS and CBR of biopolymer-stabilized soils, with finite element analysis of the UCS–CBR link. Geotechnical and Geological Engineering. Muhammed Yeasin Arafat, Department of Civil Engineering, Bangladesh University of Engineering and Technology (BUET), Dhaka 1000, Bangladesh; yeasinarafat1804177@gmail.com

This file contains Texts S1–S4, Tables S18–S22 and Figs. S4 and S5. [AUTHOR: merge the existing Tables S1–S17 and Figs. S1–S3 of the original supplement into this file before submission.]

## Text S1 Robustness of the cross-study results (full description)

Seeds. Across the three seed sets, pooled UCS R² of the networks varies by 0.08–0.37. Single-seed results can therefore suggest rankings that do not hold.

Nested selection. Selecting the model and its hyperparameters inside each fold gave R² = 0.15 for UCS and −0.12 for CBR, and the selected model changed between folds.

Single-study influence. Removing one study from the evaluation moved the UCS R² of SVR between −0.29 and 0.25, with S20 the most influential study.

Missing-value indicators. Removing them lowered the pooled UCS R² of SVR from 0.15 to −0.09. The paired per-study RMSE changed by only +0.003 (CI −0.119 to 0.108), so the pooled value depends on a few influential studies. Tree models were consistently worse without the indicators (XGBoost +0.074; CI 0.008 to 0.140).

Feature order. Reversing or permuting the input order changed the UCS R² of the CNN, LSTM and hybrid B by as much as a change of seed. Across four orders the ranges were −0.24 to 0.22 (CNN), −0.27 to 0.09 (LSTM) and 0.09 to 0.21 (hybrid B), and no order was consistently best.

Grouping S02 and S08. Holding out the two studies that share authors together changed the pooled R² of the classical models by at most 0.05 and the mean per-study RMSE by at most 0.056 log units. None of the conclusions changed.

Other checks. Excluding the censored CBR values changed R² by −0.09 to +0.01. Bounding the predictions changed at most 0.5% of them.

## Text S2 Treatment effects

The median UCS improvement over the matched control was 112% for the biopolymer alone (27 records from 8 studies; 95% CI 18–269%), 139% for the second additive alone (28 records from 8 studies; CI −10 to 175%) and 95% for combinations (136 records from 15 studies; CI 65–194%) (Fig. S2 in Online Resource 1). The intervals overlap widely and the three states come mostly from different studies, so no ranking of treatment states is supported. Among the 30 strictly matched UCS contrasts from four studies, clear synergy is rare (7–13% for any additive band). The antagonistic share, however, depends on the band chosen (37–67%), so it is not a robust finding.

The relative improvement index was RII = 100 (Y − Y0)/Y0, where Y and Y0 = strength of the treated mix and of its untreated control of the same study and curing age, with 95% study-cluster bootstrap intervals (5,000 resamples).

## Text S3 Finite element methodology (full description)

Both tests were modeled in OpenSeesPy 3.8 (McKenna 2011; Zhu et al. 2018) with eight-node stabilized single-point bricks (SSPbrick), on quarter models with two symmetry planes meshed as O-grid cylinders.

UCS model. The specimen has a diameter D = 100 mm and a height H = 200 mm (H/D = 2, as in ASTM D2166; ASTM 2016). The rigid platens are either smooth or fully bonded (rough), and loading is by displacement control to 3% axial strain.

CBR model. The mold diameter is 152 mm, the soil height 116.4 mm (ASTM D1883; ASTM 2021) and the plunger diameter 49.6 mm. The base is fixed, the rigid mold wall smooth and the rigid plunger rough, and a 2.5 kPa surcharge acts on the annulus. Penetration is applied to 5.1 mm, and CBR is evaluated with the standard forces of BS 1377-4 (BSI 1990) as

CBR=100 max​P2.513.24 kN, P5.019.96 kN	(S1)

where P2.5 and P5.0 = plunger loads at 2.5 and 5.0 mm penetration. The forces of IS 2720 (Part 16) (BIS 1987) and ASTM D1883 (ASTM 2021) were also applied to the stored curves; they change CBR by a median of −0.9% and +1.6% (Table 6), so the choice of standard does not affect the conclusions.

Solver and convergence. Krylov–Newton iterations were used with a force-unbalance tolerance of 10−3 kN and automatic sub-stepping. A run was accepted only if every step converged; failed runs were recorded and excluded.

Soil model. The soil was represented by Drucker–Prager plasticity (Drucker and Prager 1952) with associated flow. It was perfectly plastic in the UCS runs and had a small linear hardening modulus Hp = 0.005E in the CBR runs to aid convergence. The Mohr–Coulomb parameters (c, φ) were mapped either to the outer cone, which matches triaxial compression and is used unless stated otherwise,

αDP=2sinϕ3 3−sinϕ,  kDP=6ccosϕ3 3−sinϕ	(S2)

or to the inscribed inner cone,

αDP=tanϕ9+12tan2ϕ,  kDP=3c9+12tan2ϕ	(S3)

Verification, calibration and validation are kept separate (Oberkampf and Roy 2010). For code verification, the smooth-platen UCS model was compared with the closed-form Mohr–Coulomb strength

q=2ccosϕ1−sinϕ	(S4)

Numerical verification consisted of mesh refinement studies, and calibration refers to fitting the parameters of the UCS-to-CBR link to measured UCS–CBR pairs. Validation would require independent measured stress–strain or load–penetration curves. The database contains none, so the FE models were not validated against experiments.

Spatial convergence of the CBR model was assessed with four uniform refinement levels: L0 (168 elements, 273 nodes), L1 (456 elements, 648 nodes; the campaign mesh), L2 (1,248 elements, 1,612 nodes) and L3 (2,640 elements, 3,230 nodes). Each model has three degrees of freedom per node. These levels were run for a representative case (c = 60 kPa, φ = 25°, E = 60 MPa) and four further parameter sets spanning the design. The apparent order of convergence, the Richardson-extrapolated value and the grid convergence index (GCI, safety factor 1.25) were computed from the three finest levels (Roache 1994; Celik et al. 2008). Spatial convergence was accepted only for a GCI of 5% or less with monotone behavior.

To test whether the slow convergence is caused by the singularity at the plunger edge, a second mesh family with the same element counts was graded toward the edge. Its radial element size at the edge is 0.08–0.24 mm, against 1.45–2.42 mm in the uniform meshes, and the vertical grading is finer beneath the surface. The sensitivity of the representative CBR case to dilation, yield-surface matching, hardening, surcharge, specimen height, plunger roughness and load-step size was evaluated on the campaign mesh.

The simulation campaign comprised 154 runs: Latin-hypercube designs for UCS (44 runs) and CBR (100 runs) over c = 5–4,000 kPa, φ = 10–40°, E = 5–2,000 MPa and ν = 0.25–0.40, plus verification and mesh runs. Of these, 147 converged. A further 80 runs covered the mesh, sensitivity, graded-mesh and non-associated-flow studies.

A Gaussian-process surrogate was fitted to the 76 converged outer-cone CBR runs on mesh L1, so it reproduces the L1 model, including its mesh bias. In the UCS-to-CBR link, a UCS value q is converted to cohesion and stiffness through

c=q 1−sinϕ2cosϕ,  E=κ q	(S5)

and the surrogate returns CBRFEM. The parameters φ (10–40°) and κ (10–1,000) were calibrated by grid search on matched UCS–CBR pairs, always excluding the held-out study. For numerical-informed CBR networks, the FE estimate was added as an input or as a soft target, using UCS predictions made without the held-out study and without each training row’s own study.

## Text S4 Axisymmetric reference model of the CBR test

Purpose. The 3D CBR meshes did not reach the asymptotic range (main text, Sect. 3.8). Because the CBR test is axisymmetric in geometry and loading, an independent axisymmetric model was written to obtain spatially converged reference values and to quantify the discretization bias of the 3D campaign mesh. The code is provided in Online Resource 2 (files axicbr.py, verify.py, verify_ucs.py, cbr_mesh.py, gci.py and lhs.py).

Geometry and boundary conditions. Mold radius 76 mm, specimen height 116.4 mm, plunger radius 24.8 mm. The base is fixed; the mold wall is smooth (radial displacement fixed, vertical displacement free); the axis carries the symmetry condition. The plunger is rigid and rough: the nodes under it receive the same prescribed vertical displacement and zero radial displacement. A surcharge of 2.5 kPa acts on the annulus outside the plunger. Penetration is applied in displacement steps of 0.02 mm to 0.2 mm, 0.05 mm to 2.5 mm and 0.1 mm to 5.1 mm; a step that does not converge is halved automatically. The plunger force is the sum of the reactions at the plunger nodes, and CBR = 100 max(P2.5/13.24 kN, P5.0/19.96 kN), as in the 3D model.

Elements and material. Eight-node serendipity quadrilaterals with 2 × 2 Gauss integration in the axisymmetric formulation. The Drucker–Prager yield function f = αDP I1 + √J2 − kDP uses the outer cone matched to Mohr–Coulomb in triaxial compression, with associated flow. The return mapping is solved in closed form for the cone and the apex, and the consistent tangent is used in a Newton iteration with a tangent predictor and a backtracking line search (convergence when the residual norm falls below 10−7 of the reaction-force norm). The linear hardening modulus of kDP is Hk = 0.005 E (αDP + 1/√3)2, which gives a hardening slope of 0.005E in uniaxial compression. Because the hardening definition of the OpenSees material used in the 3D model may differ, absolute values can differ slightly between the two codes, mainly for weak soils, which are the most sensitive to hardening (Table 6).

Meshes. Each level is a self-similar refinement of a base mesh of 140 elements graded toward the plunger edge (radial grading ratio 1.6) and toward the surface (vertical ratio 1.8). Level n divides every base element into n × n elements, so levels 1, 2, 4 and 8 have 140, 560, 2,240 and 8,960 elements (815 to 53,567 free degrees of freedom) and a constant refinement ratio of 2. Richardson extrapolation and the grid convergence index (safety factor 1.25) were computed from levels 2, 4 and 8.

Verification. (i) Homogeneous UCS with smooth platens: the computed peak equals the closed-form Mohr–Coulomb strength to within 0.001% (displayed as 0.000% in Table 6) for φ = 10°, 20°, 30° and 40°; the inner cone gives 0.637 times this value. (ii) Elastic rigid circular punch: the extrapolated load is 1.052 times the half-space solution of Sneddon (1965) on a domain of 20 punch radii and 1.024 times on 60 radii; the excess decreases toward 1 as the domain grows, as expected for a finite domain.

Sensitivity design. Sixty Latin-hypercube samples (seed 2026) over log c (5–800 kPa), φ (10°–40°), log E (5–400 MPa) and ν (0.25–0.40) were run on mesh level 2. At the five reference sets, level 2 exceeds the converged value by a nearly constant 4.5–6.4%, so this level preserves the ranking and the sensitivities. Standardized regression coefficients were obtained by least squares on standardized inputs and standardized log10 CBR.

Converged-mesh sensitivity (level 4, five reference sets). Smooth instead of rough plunger: −1.9%, −0.7%, −0.2%, −1.4% and −2.1% (stiff frictional, strong stiff, cohesive soft, representative, weak). Hardening 0.001E / 0.02E instead of 0.005E: −0.9/+1.3%, −3.6/+4.5%, −2.3/+5.3%, −6.4/+26.7% and −20.8/+61.7%. Inner cone: −69.3%, −61.0%, −29.4%, −55.3% and −34.1%.

Converged surrogate for the UCS–CBR link. 252 runs on mesh level 2 over UCS q = 60–3,000 kPa (six log-spaced values), φ = 10°–40° (step 5°) and κ = E/q = 3–150 (six log-spaced values), with c from Eq. (5) of the main text and ν = 0.3; all runs converged. log10 CBR was interpolated with cubic splines in (log q, φ, log κ) and divided by the mean level-2 bias factor (1.051). The calibration objective (RMSE of log10 CBR over the pairs) was evaluated on a grid of φ = 10°–40° (step 1°) and 60 log-spaced κ values from 3 to 150.

Reconstructed UCS–CBR pairs. Each unsoaked or condition-not-stated CBR record was matched to the UCS records of the same study and mix (biopolymer, second additive and both doses); where several curing times were available, the UCS records at the nearest curing time were averaged. This gives 86 pairs from 7 studies (S01, S02, S03, S05, S08, S11, S18), 69 of them at identical curing times. The original analysis used 91 pairs; the difference comes from the pairing rule, which was not deposited.

AI assistance. The code of this model was written with the help of a generative AI assistant (main text, Sect. 2.6); all verification results above were produced by running it. [AUTHOR TO CONFIRM]

Sensitivity result. Standardized regression coefficients of log10 CBR (n = 60, R² = 0.89; 95% bootstrap intervals over 2,000 resamples in parentheses): log E 0.77 (0.67–0.87); φ 0.47 (0.36–0.57); log c 0.28 (0.17–0.38); ν 0.02 (−0.07 to 0.11). The computed CBR ranged from 4.2% to 610%; all 60 runs converged.

Table S18 Data-quality and harmonization audit

| Check | Finding | Records affected | Rule applied | Final status |
| --- | --- | --- | --- | --- |
| Counts and attribution | 397 UCS / 19 studies; 163 CBR / 10 studies; 20 studies; counts agree with the study inventory | All | — | Verified |
| Source identification | 18 of 20 database DOIs resolve to matching publications; S06 DOI malformed; S20 without DOI | S06: 2 CBR; S20: 30 UCS, 10 CBR | Cited through the database | Retained |
| Identical records | 4 UCS and 14 CBR rows with identical inputs and target | As stated | Treated as reported replicate values | Retained |
| Replicate groups | 38 UCS groups (94 rows) and 32 CBR groups (90 rows) with identical inputs | As stated | Never split across folds; excluded from evaluation when used as local tests | Retained |
| Overlap between studies | No mix and no soil characterization shared by two studies; S02 and S08 share two authors | S02 + S08: 21 UCS, 14 CBR | Sensitivity analysis with S02 and S08 held out together | Retained |
| Ranges and consistency | No violations (Gs 2.0–3.2, LL ≤ 200%, OMC ≤ 60%, MDD 1,000–2,400 kg/m³, doses 0–100%, PL ≤ LL) | 0 | — | Verified |
| Units | Untreated OMC reported as a fraction | 60 UCS | Multiplied by 100 (database QC log) | Corrected |
| Column swap | Treated MDD and OMC swapped | 2 CBR | Swapped back (database QC log); variables not used as inputs | Corrected |
| Curing | Curing given only as “optimum moisture content” | 4 CBR (S17) | Excluded | Excluded |
| Curing inferred from text | Curing time inferred (e.g., “after 96 h”) | 9 CBR | Retained; flagged in database | Retained |
| Censoring | CBR reported as “<1” | 5 CBR (S05) | Stored as 1.0; sensitivity analysis excludes them | Retained |
| Dose not reported | Lignin stabilizer dose missing | 18 UCS, 36 CBR (S05) | Coded as treated with a flag | Retained |
| Treatment indicators | Additive indicators set for zero-dose controls | 57 UCS, 54 CBR | Treatment state defined from doses | Corrected in derived tables |
| Missing soil properties | Gs 27%, D50 19%, OMC and MDD 30% of UCS records missing (whole studies); grain-size fractions 41–44% missing | 3–10 studies per variable | Median imputation and indicators fitted within folds; fractions excluded | Retained |
| Protocol variables | Test standard not recorded for any study; moisture at test missing for 93% of UCS and all CBR records; CBR penetration recorded for 63 of 163; CBR condition not stated for 24 | Most records | Cannot be harmonized; condition-not-stated indicator used | Limitation |

Note: MDD = maximum dry density; OMC = optimum moisture content; QC = quality control; — = not applicable.

Table S19 Models and validation framework

| Item | Specification |
| --- | --- |
| Models | Training mean; Ridge (α = 1); SVR (RBF kernel); GPR (Matérn 5/2 + white noise); random forest; XGBoost; LightGBM; ANN (64–32); DNN (256–128–64–32, dropout 0.2); 1-D CNN (2 × 32 filters); LSTM, GRU (32 units); hybrid A (ANN–DNN–CNN); hybrid B (LSTM–GRU–DNN); hybrid B + attention |
| Preprocessing (fitted on training rows of each fold only) | Median imputation; missing-value indicators; standardization; clipping at ±4 SD; prediction bound = training-target range ± 0.3 log units (results also computed unbounded) |
| Network training | PyTorch, full-batch AdamW (learning rate 3 × 10−3, weight decay 10−4), ≤ 2,000 epochs, early stopping (patience 150) on 15% of training studies (LOSO) or rows (random split); each prediction averages two network seeds |
| Seeds | Three seed sets for networks ({0,1}, {2,3}, {4,5}) and tree ensembles (0–2) |
| Tuning | Fixed settings for the main comparison; nested selection of model family and hyperparameters (Ridge α; SVR C, ε; XGBoost depth and trees; DNN learning rate and weight decay) by 4-fold study-grouped inner CV |
| Validation designs | (A) random 5-fold row split, five split seeds (interpolation-oriented); (B) leave-one-study-out (LOSO); (C) nested grouped selection within LOSO; (D) leave-one-group-out with S02 and S08 merged |
| Baselines | Training mean and Ridge, evaluated on exactly the same held-out records |
| Metrics | Pooled R², RMSE and bias on the log10 target; RMSE and MAE in kPa or %; median absolute percentage error; RMSE of each held-out study (log units) and its mean across studies |
| Comparison | Paired per-study RMSE differences against the training mean; 95% study-bootstrap CI (5,000 resamples); Wilcoxon signed-rank test with Holm adjustment over 14 models; standardized effect size dz |

Note: ANN = artificial neural network; CNN = convolutional neural network; DNN = deep neural network; GPR = Gaussian-process regression; GRU = gated recurrent unit; LSTM = long short-term memory network; MAE = mean absolute error; RBF = radial basis function; RMSE = root-mean-square error; SVR = support vector regression.

Table S20 Robustness checks and limitations

| Issue | Test | Result | Implication |
| --- | --- | --- | --- |
| Random-split optimism | Random 5-fold, five split seeds, vs LOSO | R² 0.90–0.96 (UCS) vs ≤ 0.15 | Study-wise validation required |
| Seed variability | Three seed sets | Network UCS R² varies by 0.08–0.37 | Single-seed rankings unreliable |
| Model selection leakage | Nested study-grouped selection | R² 0.15 (UCS), −0.12 (CBR) | No gain over fixed models |
| Influential studies | Remove each study from the evaluation | SVR UCS R² −0.29 to 0.25 | Pooled R² fragile |
| Missing-value indicators | Remove all indicators | Pooled SVR R² 0.15 → −0.09; per-study RMSE +0.003 | Pooled UCS result not robust |
| Feature order | Four orders for CNN, LSTM, hybrid B | Effect as large as seed effect | Sequence branches not justified |
| Related studies | S02 and S08 held out together | Classical R² change ≤ 0.05 | Conclusions unchanged |
| Censored CBR | Exclude 5 values | ΔR² −0.09 to +0.01 | Conclusions unchanged |
| Multiplicity | Holm adjustment over 14 models | All adjusted p = 1.0 | No model better than the mean |
| Local calibration | Simulation (300 draws) and one external study | Gain in simulation; no gain from untreated control externally | Exploratory; confirm with new tests |
| CBR mesh | Four uniform levels, five parameter sets; graded meshes | GCI 3.6–35%; L1 bias 9–91% | Absolute FE CBR not quantitative |
| Constitutive assumptions | Dilation, cone, hardening | CBR −50% to +40% | Factor 1.5–2 model-form uncertainty |
| FE validation | Not possible: no measured curves | — | FE numerically verified only |
| Number of studies | 19 UCS, 10 CBR | Low power | Equivalence cannot be claimed |

Fig. S4 Measured versus leave-one-study-out predicted values (mean over seed sets) for SVR: (a) UCS, 19 held-out studies; and (b) CBR, 10 held-out studies. Symbols distinguish studies; the solid line is equality, and dashed lines mark a factor of 2.

Fig. S5 3D UCS model: (a) von Mises stress at 2% axial strain with rough platens (c = 150 kPa, φ = 25°, E = 100 MPa; deformation magnified three times); (b) normalized stress–strain curves of the representative case and of the 32 Latin-hypercube runs; and (c) ratio of computed to closed-form strength for smooth and rough platens at three mesh levels.

Table S21 Mixed-effects and restricted-input baselines under leave-one-study-out validation, and mixed-model calibration

| Target | Predictor | Inputs | Mean per-study RMSE | LOSO R² | Δ vs training mean (95% CI) | Studies better | p (Wilcoxon) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UCS | Training mean | — | 0.413 | −0.10 | — | — | — |
| UCS | Random-intercept LMM | none | 0.420 | −0.09 | +0.007 (−0.008 to 0.023) | 8/19 | 0.40 |
| UCS | Random-intercept LMM | doses, curing, condition | 0.393 | −0.03 | −0.020 (−0.094 to 0.065) | 11/19 | 0.29 |
| UCS | Random-intercept LMM | + family indicators | 0.589 | −2.21 | +0.176 (0.030 to 0.343) | 5/19 | 0.04 |
| UCS | Random-intercept LMM | + families, soil properties | 2.702 | −79.32 | +2.289 (0.959 to 3.978) | 3/19 | <0.001 |
| UCS | SVR (re-implemented) | all (one-hot families, indicators) | 0.387 | 0.21 | −0.026 (−0.094 to 0.043) | 12/19 | 0.37 |
| UCS | SVR (re-implemented) | within-study inputs + families | 0.482 | −0.29 | +0.069 (−0.022 to 0.160) | 7/19 | 0.23 |
| UCS | Ridge | within-study inputs + families | 0.566 | −1.14 | +0.153 (0.042 to 0.266) | 6/19 | 0.03 |
| CBR | Training mean | — | 0.392 | −0.10 | — | — | — |
| CBR | Random-intercept LMM | none | 0.397 | −0.06 | +0.006 (−0.003 to 0.013) | 2/10 | 0.16 |
| CBR | Random-intercept LMM | doses, curing, condition | 0.476 | −0.25 | +0.085 (−0.024 to 0.205) | 4/10 | 0.32 |
| CBR | Random-intercept LMM | + family indicators | 0.540 | −0.64 | +0.148 (0.028 to 0.266) | 3/10 | 0.08 |
| CBR | Random-intercept LMM | + families, soil properties | 0.552 | −0.70 | +0.161 (0.039 to 0.296) | 2/10 | 0.06 |
| CBR | SVR (re-implemented) | all (one-hot families, indicators) | 0.424 | −0.03 | +0.032 (−0.028 to 0.102) | 5/10 | 0.43 |
| CBR | SVR (re-implemented) | within-study inputs + families | 0.439 | −0.01 | +0.047 (−0.041 to 0.140) | 5/10 | 0.43 |
| CBR | Ridge | within-study inputs + families | 0.521 | −0.53 | +0.130 (0.003 to 0.260) | 4/10 | 0.19 |

| Target | k | Local mean | LMM + full shift | LMM + BLUP shift | SVR + full shift | Studies |
| --- | --- | --- | --- | --- | --- | --- |
| UCS | 1 | 0.263 | 0.230 | 0.226 | 0.228 | 18 |
| UCS | 2 | 0.224 | 0.197 | 0.196 | 0.195 | 18 |
| UCS | 3 | 0.221 | 0.190 | 0.190 | 0.187 | 17 |
| CBR | 1 | 0.343 | 0.425 | 0.408 | 0.323 | 9 |
| CBR | 2 | 0.308 | 0.387 | 0.382 | 0.291 | 9 |
| CBR | 3 | 0.283 | 0.362 | 0.361 | 0.268 | 9 |

Per-study RMSE in log10 units. LMM = linear mixed-effects model with a random intercept for each study, fitted by REML in each fold; for a held-out study, the prediction is the fixed part. The SVR was re-implemented (RBF kernel, C = 1, ε = 0.1) because the original model code is not in the deposited database; its values therefore differ from Table 3. Within-study inputs: biopolymer dose, missing-dose flag, second-additive dose, total dose, curing days, ln(1 + days), total dose × ln(1 + days) and, for CBR, the soaked and condition-not-stated indicators. Calibration: k local mixes (mix = study, doses and additives) drawn 300 times per study; studies with no more than k mixes are excluded. BLUP shift = full shift × n τ2/(n τ2 + σ2). — = not applicable.

Table S22 External test of the converged UCS–CBR link on Banne et al. (2024): unsoaked CBR (%) of four laterite soils with 0–3% guar gum

| Soil | Guar gum (%) | c (kPa) | φ (°) | q (kPa) | κ | Measured CBR | FE chain | Power law |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 14.7 | 22.81 | 44 | 37.3 | 2.45 | 2.33 | 9.2 |
| A | 1 | 23.5 | 26.35 | 76 | 34.9 | 3.59 | 4.02 | 11.4 |
| A | 2 | 28.4 | 25.71 | 91 | 34.9 | 5.47 | 4.76 | 12.1 |
| A | 3 | 26.5 | 25.96 | 85 | 34.9 | 5.55 | 4.47 | 11.8 |
| B | 0 | 14.7 | 22.81 | 44 | 37.3 | 2.42 | 2.33 | 9.2 |
| B | 1 | 26.5 | 28.23 | 89 | 34.9 | 3.92 | 4.82 | 12.0 |
| B | 2 | 30.4 | 29.29 | 104 | 34.9 | 5.79 | 5.72 | 12.8 |
| B | 3 | 24.5 | 31.92 | 88 | 32.6 | 5.93 | 4.67 | 12.0 |
| C | 0 | 14.7 | 22.81 | 44 | 37.3 | 2.43 | 2.33 | 9.2 |
| C | 1 | 32.4 | 29.46 | 111 | 34.9 | 3.83 | 6.12 | 13.1 |
| C | 2 | 34.3 | 31.04 | 121 | 32.6 | 5.93 | 6.39 | 13.6 |
| C | 3 | 32.4 | 34.87 | 124 | 32.6 | 6.10 | 6.68 | 13.7 |
| D | 0 | 14.7 | 22.81 | 44 | 37.3 | 2.43 | 2.33 | 9.2 |
| D | 1 | 30.4 | 29.24 | 104 | 34.9 | 3.74 | 5.71 | 12.8 |
| D | 2 | 32.4 | 30.41 | 113 | 34.9 | 6.10 | 6.30 | 13.2 |
| D | 3 | 31.4 | 31.29 | 112 | 32.6 | 6.24 | 5.88 | 13.1 |
| RMSE (log10) |  |  |  |  |  |  | 0.084 | 0.455 |
| Mean bias (log10) |  |  |  |  |  |  | +0.015 | +0.442 |

c and φ from direct shear tests (Banne et al. 2024, Table 3; 1 kg/cm2 = 98.07 kPa); measured CBR digitized from their Fig. 10 (unsoaked bars). q = 2c cos φ/(1 − sin φ), Eq. (4). κ = value minimizing the calibration objective on the 86 database pairs at the given φ; FE chain = axisymmetric model, level 2, corrected for the level-2 bias. Power law fitted to the 86 database pairs, applied to q. Training mean of the pairs: 21.2% (RMSE 0.719). With κ at the lower and upper ends of the near-optimal band, the FE RMSE is 0.122 and 0.154. The untreated soil has the same c and φ for the four soil types.

# References

Banne SP, Kulkarni S, Baldovino JA (2024) Effect of guar gum content on the mechanical properties of laterite soil for subgrade soil application. Polymers 16(15):2202. https://doi.org/10.3390/polym16152202

Laird NM, Ware JH (1982) Random-effects models for longitudinal data. Biometrics 38(4):963–974. https://doi.org/10.2307/2529876

Sneddon IN (1965) The relation between load and penetration in the axisymmetric Boussinesq problem for a punch of arbitrary profile. Int J Eng Sci 3(1):47–57. https://doi.org/10.1016/0020-7225(65)90019-4